DS001A Driver Monitoring Camera

User Guide and Technical Specifications

The DS001A is an in-cab camera with a built-in driver monitoring algorithm. It monitors driver fatigue and driving behaviour, provides voice alerts through a connected speaker, and outputs alarm signals for integration with a vehicle recorder or other equipment.

Camera dimensions 63 × 50.5 × 104 mm

1 Driver monitoring functions

| Function | Detection |
| --- | --- |
| Driver fatigue | Detects signs of fatigue, including eye closure. |
| Seat belt unfastened | Detects when the driver is not wearing a seat belt. |
| Drinking water | Detects the driver drinking water. |
| Smoking | Detects the driver smoking. |
| Driver distraction | Detects inattentive driving. |
| Phone calls | Detects the driver making or receiving a phone call. |
| Abnormal posture | Provides an alert when abnormal driver posture is detected. |

When an event is detected, the camera provides a voice alert, a 5 V high-level I/O alarm signal and an alarm message through the serial interface.

2 System connections

The standard system component is the DS001A driver monitoring camera. The cable assembly provides the following six connections.

Cable assembly and connector numbers

| No. | Connection | Purpose |
| --- | --- | --- |
| 1 | Speaker interface | Connects to an in-cab speaker for voice alerts. |
| 2 | Video connector | Provides camera power, alarm audio output and AHD 720P video output. |
| 3 | RS485 serial wires | Outputs alarm messages for integration with a vehicle recorder or multiple BSD cameras. |
| 4 | DC power connector | DC 12-36 V power connection for the external audible and visual alarm. |
| 5 | Audible and visual alarm connector | Provides power and a control signal for an external audible and visual alarm. |
| 6 | GPS module interface | Connects an external GPS module; GPS speed can be used to filter alarms. |

Alarm outputs

The camera supports one 5 V I/O alarm output and RS485 alarm data output. The video connector also carries alarm audio. Voice prompts are delivered through a connected in-cab speaker.

3 Interface definitions

3.1 Speaker interface

Connect an in-cab speaker to connector 1 for voice alerts to the driver.

3.2 Video connector

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | Power positive | 12 V |
| 2 | Power negative | Ground |
| 3 | Audio output | Alarm audio output |
| 4 | Video output | AHD 720P signal output |

3.3 RS485 serial interface

| Wire | Signal |
| --- | --- |
| Green | RS485 A |
| White/green | RS485 B |

Serial protocol output supports connection to a vehicle recorder or multiple BSD cameras. Refer to the camera communication protocol for integration details.

3.4 DC power connector

Connector 4 provides the DC 12-36 V power connection for the external audible and visual alarm.

3.5 Audible and visual alarm connector

| Pin | Signal | Description |
| --- | --- | --- |
| 1 | Power positive | 12-36 V |
| 2 | Power negative | Ground |
| 3 | Not connected | Unused |
| 4 | Alarm output | Controls the external audible and visual alarm |

3.6 GPS module interface

Connector 6 is for an external GPS positioning module. Vehicle speed from GPS can be used to filter alarm events.

Camera power supply: DC 12 V ±10%. The 12-36 V rating above applies to the external alarm connections.

4 Technical specifications

| Parameter | Specification |
| --- | --- |
| Model | DS001A |
| CPU | ARM Cortex-A7 |
| NPU | 0.6 TOPS |
| Image sensor | 1/3-inch CMOS, AHD 720P |
| Frame rate | 30 fps |
| Video standard | PAL |
| Effective resolution | 1080 (H) × 720 (V) |
| Effective pixels | 1.0 megapixel |
| Vertical sync frequency | 50 Hz / 60 Hz |
| Minimum illumination | Colour: 0.05 lux at F1.2 |
| Video output level | 1.0 Vp-p, 75 Ω |
| Automatic gain control | Auto |
| Signal-to-noise ratio | >50 dB |
| White balance | Auto |
| Backlight compensation | Auto |
| Lens | 2.8 mm, F2.0 |
| Field of view | 100° |
| Night vision | Starlight night vision |
| Audio | None |
| Built-in algorithm | Driver fatigue monitoring (DSM) algorithm |
| Alarm interface | 1 I/O alarm output, 5 V |
| Serial interface | RS485 |
| Video interface | AHD signal |
| GPS interface | External GPS positioning module |
| Speaker interface | In-cab speaker interface for voice alerts |
| Current | 200 mA ±10% |
| Camera power supply | DC 12 V ±10% |
| Water protection | Not waterproof |
| Operating conditions | -40°C to +70°C; 95% RH maximum |
| Storage conditions | -30°C to +80°C; 95% RH maximum |
| Dimensions | 63 × 50.5 × 104 mm |

5 Reference information

Purpose and scope

This document describes the DS001A camera for monitoring driver fatigue and driving behaviour in vehicles. It covers system functions, interfaces and technical specifications for use during system design, development, integration and testing.

DSM refers to the driver fatigue monitoring system in this document.

Reference document

Driver Fatigue Monitoring System Functional Specification.

Design and test standards

The following references apply to both design and testing.

| Reference | Title | Edition |
| --- | --- | --- |
| JT/T 883 | Technical requirements and test methods for driving hazard warning systems on commercial vehicles | 2014 |
| GB/T 2423.1 | Environmental testing for electrical and electronic products - Test A: Cold | 2008 |
| GB/T 2423.2 | Environmental testing for electrical and electronic products - Test B: Dry heat | 2008 |
| GB/T 2423.5 | Environmental testing for electrical and electronic products - Test Ea and guidance: Shock | 2008 |
| GB/T 2423.17 | Environmental testing for electrical and electronic products - Test Ka: Salt mist | 2008 |
| GB/T 2423.56 | Environmental testing for electrical and electronic products - Test Fh: Broadband random vibration | 2008 |
| GB/T 19951 | Road vehicles - Test methods for electrical disturbances from electrostatic discharge | 2008 |

Communication protocol

Refer to the DSM Camera Communication Protocol document for serial communication and alarm message definitions.